Conveyor Length (CL)

1,000 ~ 2,000mm

Conveyor Height (CH)

500~1,500mm (+50 & -20mm)

Conveyor Speed

Fixed (22M/Min.) & Variable (Max. 44M/Min.)

Standard Rejecting System

Belt Stops, Audible Alarm & Lamp (Auto Rejecter available as option)

Preset Numbers

100 items

Outputs

Relay (5A) / 1A1B 2EA, DC24V SOL, AC220V 0.2A

Power Supply

Single Phase (110 to 220V) & Three Phase (220 to 380V and etc.,), Free Cycle (50/60Hz) & Fuse (3A)

Operating Temperature 
 	0˚C to 50˚C (advised to use the machine under the normal condition)

High Performance for Metal Detection

• Closed Coil System for perfect detection

• High quality filter for more precise detection

• Auto absorption function for vibration, impact, noise and etc.,

• Dust Proof & Water Proof Standard (IP66) authentication

Multi Frequency Operation

• Supporting multi frequency operation mode that can measure various types of products with the correct operating frequency for maximum contaminant sensitivity in seconds

Phase Tracking Function

• Optimum detecting level is maintained by automatically coping with products of which phase is changed minutely, so detection is rather difficult such as frozen food.

Full Color Touch Screen (7 inch TFT LCD)

• An intuitive and convenient full color touch screen, containing regional languages and user manual

Digital Signal Processing Function

• The adoption of 32-bit microprocessor has added more precise operation and performance of the equipment. Software filter technology to remove ambient noise affecting performance of a metal detector has been applied.

Exterior Design considering the convenience and aesthetics

• Modern and sensuous design in which both convenience and beauty are emphasized at the same time

Connectivity to external devices

• Backing up inspection history and etc., by using a USB removable disk and reviewing the data on a PC.

• Monitoring real-time operating status for multiple metal detectors through the connective access to LAN.

• Interlocking the machine to various typed external devices through RS232C & 485 port.

Auto-setting function of the inspection items

• Auto-setting the machine for a testing product with the most suitable values even for beginners not to operate the equipment properly.

Setting function of a variety of items for convenient operation

• Up to 100 kinds of products are registered with each different setting value for convenient operation.

Manageable Metal Detecting History

• Detection history by various classifications and periods (in a daily / weekly / monthly / period-specified way)

• Data on detection history accessibility by LCD screen, USB disk and Printer (Optional).

Easy Software upgrade

• Easy software upgradability by USB and touch screen whenever the latest version is available.